StarHub 咖啡机NX教程

公开教程 · 非商业授权版

这份教程可以帮助你在你拥有或明确获授权的咖啡机与手机上，完成一次正常制作时的蓝牙通信完整保存下来，再通过重复实验判断哪些字段代表配方、参数、开始和完成状态然后复现，最终让你在自己的设备和系统上获得咖啡机的控制权。

我用的咖啡机：bincco 指挥家，别的大概也能通用

目标栈：iPhone sysdiagnose / PacketLogger / Python 3.11+ / SHA-256

授权边界：公开源代码仅为便于审阅与学习，不授予商业使用、转售、再分发、代运营、设备控制或衍生产品权利。严禁任何未经福圆童子 / StarHub 书面允许的使用。

・┈・┈・・┈・┈・・┈・┈・⊹*:ꔫ:*˖⊹・┈・┈・┈・┈・┈・┈・・┈・

人话版：跑通的5个步骤

| 1 圈定单杯 | 一次只做一杯；记录配方、设置、北京时间和开始/结束时刻。 |
| --- | --- |
| 2 保存原件 | 立刻导出 sysdiagnose/PacketLogger，算 SHA-256；原件只读，分析副本脱敏。 |
| 3 还原链路 | 按连接会话重组 ACL/L2CAP，再识别 ATT 写入、响应和通知；方向靠已知动作校准。 |
| 4 做变量对照 | 同一配方重复 2–3 次，再只改一个变量；区分稳定字节、变化字节和噪声。 |
| 5 离线验收 | 先回放证据，再谈控制；真实写入默认关闭，未知帧、断线和超时全部失败关闭。 |

完成标准

原始证据可复核；每个结论都能指向时间窗与包序号；未知字段保持 unknown；旧通知、不完整写入、断线、超时和设备未回到待机都不会被标记为成功。

NEVER：绕过鉴权/加密、扫描他人设备、尝试 DFU/重置/未知配置、对不确定写入自动重试，或公开真实地址、密钥和私人日志。

下一页开始：machine-readable YAML，可直接丢给机。

MACHINE-READABLE 01 / policy + workflow

--- document: id: starhub-coffee-capture-tutorial-v1 title: "StarHub 咖啡机逆向抓包教程" language: zh-CN edition: "公开教程 / 非商业授权版" watermark: "STARHUB-NC-2026-COFFEE-CAPTURE" copyright: "Copyright (c) 2026 StarHub / 翘翘" license: source_visible: true commercial_use: prohibited redistribution: prohibited derivative_commercial_product: prohibited unauthorized_use: prohibited permitted_use: - personal_study - written_authorized_noncommercial_research notice: >- 仅限个人学习及书面授权的非商业用途；严禁商业使用、转售、 再分发、代运营或任何未经翘翘 / StarHub 书面允许的使用。 scope: allowed_target: "自有设备或已取得明确书面授权的设备" transport: "Bluetooth LE / ATT" capture_source: "iPhone sysdiagnose / PacketLogger" output: "可复核证据 + 脱敏后的离线协议描述" excluded: - bypass_authentication - bypass_encryption_or_drm - scan_or_control_third_party_devices - firmware_dfu_reset_or_unknown_configuration - publish_credentials_or_real_device_identifiers principles: - one_cup_one_documented_session - preserve_original_evidence_read_only - hash_before_analysis - calibrate_direction_with_known_action - reassemble_before_decode - change_one_variable_at_a_time - unknown_stays_unknown - offline_replay_before_device_write - fail_closed_on_ambiguity workflow: - id: plan action: "定义唯一变量与预期观察窗口" - id: capture action: "完成正常制作并立即导出原始日志" - id: preserve action: "计算 SHA-256；原件只读；分析副本脱敏" - id: parse action: "连接会话 -> ACL 重组 -> L2CAP -> ATT" - id: correlate action: "写入/响应/通知与人工时间线对齐" - id: diff action: "重复样本 + 单变量样本做字段对照" - id: replay action: "离线校验顺序、响应、完成与待机回归" - id: gate action: "满足 acceptance 后才允许单次、人工在场的设备测试" stop_conditions: - missing_original_hash - ambiguous_peer_or_direction - fragmented_acl_not_reassembled - unknown_write_or_unknown_handle - timeout_disconnect_or_uncertain_write_result - device_not_back_to_idle

MACHINE-READABLE 02 / capture_manifest

--- capture_manifest: schema_version: 1 watermark: "STARHUB-NC-2026-COFFEE-CAPTURE" case_id: "YYYYMMDD-recipe-NN" authorization_ref: "<written-authorization-or-owner-statement>" timezone: "Asia/Shanghai" operator: "<name-or-pseudonym>" device: model: "<coffee-machine-model>" peer_id_public: "<redacted-stable-alias>" firmware: "<if-known>" phone: model: "<iphone-model>" os_build: "<ios-build>" official_app_version: "<version>" recipe: public_name: "<name>" settings: size: "<value>" strength: "<value>" temperature: "<value>" changed_variable: "<none-or-one-field>" timeline_bj: connect: "YYYY-MM-DDTHH:MM:SS.sss+08:00" tap_start: "YYYY-MM-DDTHH:MM:SS.sss+08:00" physical_start: "YYYY-MM-DDTHH:MM:SS.sss+08:00" physical_finish: "YYYY-MM-DDTHH:MM:SS.sss+08:00" idle_return: "YYYY-MM-DDTHH:MM:SS.sss+08:00" disconnect: "YYYY-MM-DDTHH:MM:SS.sss+08:00" observation: normal: true manual_pause: false notes: "<no-private-chat-content>" source: original_path: "<read-only-source-path>" source_sha256: "<sha256>" source_bytes: 0 bluetooth_log: "logs/Bluetooth/bluetoothd-hci-latest.pklg" analysis_copy: "<redacted-copy-path>" privacy: real_peer_address_exported: false credentials_exported: false unrelated_sessions_exported: false private_content_exported: false sampling: same_recipe_repeats: 3 single_variable_rule: true multiple_cups_in_one_sysdiagnose: >- allowed only when each time window and connection session remains attributable evidence_required: - connection_identity_and_window - gatt_service_characteristic_map - phone_to_device_writes_with_packet_index - write_responses_or_explicit_no_response_semantics - device_notifications_or_indications - fragmentation_and_reassembly_record - normal_end_and_idle_evidence evidence_labels: allowed: [observed, derived, unknown] forbidden: [guessed, inferred_from_filename_only] export_artifacts: - manifest.yaml - hashes.sha256 - peer-only.pklg - events.jsonl - field-diff.yaml - replay-report.json notes: - "不得按十六进制片段猜方向；用已知动作校准 kind/direction。" - "不得仅凭配方名称或截图猜 recipe_id。" - "一个 sysdiagnose 可覆盖多杯，但每杯必须能独立归因。"

MACHINE-READABLE 03 / offline parser

--- reference_code: language: python filename: parse_packetlogger_offline.py purpose: "只读解析 PacketLogger 容器；不连接、不扫描、不写设备" code: ¦ # STARHUB-NC-WATERMARK # Copyright (c) 2026 StarHub / 翘翘 # 仅限个人学习及书面授权的非商业用途；严禁商用和未经允许的使用。 from __future__ import annotations from dataclasses import dataclass from hashlib import sha256 from pathlib import Path import json, struct @dataclass(frozen=True) class Record: offset: int seconds: int micros: int kind: int payload: bytes def read_pklg(path: Path) -> tuple[list[Record], str]: raw = path.read_bytes() out, off = [], 0 while off < len(raw): if len(raw) - off < 13: raise ValueError("truncated_record_header") length, sec, usec = struct.unpack_from(">III", raw, off) end = off + 4 + length if length < 9 or usec >= 1_000_000 or end > len(raw): raise ValueError("invalid_packetlogger_record") out.append(Record(off, sec, usec, raw[off + 12], raw[off + 13:end])) off = end return out, sha256(raw).hexdigest() def acl_envelope(record: Record) -> dict ¦ None: if record.kind not in {2, 3} or len(record.payload) < 4: return None handle_flags, acl_len = struct.unpack_from("<HH", record.payload) if acl_len != len(record.payload) - 4: raise ValueError("acl_length_mismatch") return { "offset": record.offset, "raw_kind": record.kind, "handle": handle_flags & 0x0FFF, "pb": (handle_flags >> 12) & 0x03, "data_hex": record.payload[4:].hex(), } def export_envelopes(source: Path, target: Path) -> None: records, digest = read_pklg(source) rows = [x for r in records if (x := acl_envelope(r)) is not None] # pb=1 等 continuation 必须先按 handle+direction 重组，再解析 L2CAP/ATT。 # raw_kind 的方向先用已知动作校准；不得仅凭数值或 hex 猜方向。 target.write_text( json.dumps({"source_sha256": digest, "acl": rows}, indent=2), encoding="utf-8", ) if __name__ == "__main__": # 输入必须是脱敏后的分析副本；原件保持只读。 export_envelopes(Path("peer-only.pklg"), Path("acl-envelope.json")) parser_contract: l2cap: header: "length:uint16_le, cid:uint16_le" att_cid: 4 require_exact_length: true att_opcodes_of_interest: write_request: "0x12" write_command: "0x52" write_response: "0x13" error_response: "0x01" notification: "0x1b" indication: "0x1d" direction_rule: "calibrate_from_known_action; never infer from hex substring" frame_validation: source: "observed device-specific protocol.yaml" unknown_magic_or_checksum: "reject; do not guess"

MACHINE-READABLE 04 / diff + replay gate

--- analysis_and_replay: watermark: "STARHUB-NC-2026-COFFEE-CAPTURE" field_diff: grouping_keys: [recipe_name, settings, app_version, firmware] minimum_repeats_per_same_recipe: 2 preferred_repeats_per_same_recipe: 3 comparison: stable_bytes: "same offset and value in all same-recipe captures" variable_bytes: "change only when the documented variable changes" noise: "timestamps, counters, connection handles, transport artifacts" unknown: "insufficient evidence or conflicting observations" rule: "correlation is not semantics; label unknown until falsifiable" event_model: required_fields: [case_id, session_id, packet_index, t_relative_ms, layer, direction_calibrated, handle_or_cid, opcode, payload_hex_redacted, evidence_label] allowed_layers: [hci, acl, l2cap, att, device_frame] offline_replay: input: [events.jsonl, protocol.yaml, capture_manifest.yaml] device_io: disabled expected_sequence: - verified_recipe_write - att_response_or_verified_write_command_semantics - verified_start_write - start_ack_or_verified_notification - progress_notifications - verified_completion_notification - verified_idle_return forbidden_behavior: [fixed_delay_as_idle_proof, automatic_retry_after_uncertain_write, duplicate_brew_after_receipt_or_ui_failure, accept_old_notification_from_previous_session, execute_unknown_opcode_or_handle] device_test_gate: default_enabled: false prerequisites: [written_authorization_present, source_hash_verified, direction_calibrated, all_acl_fragments_reassembled, exact_write_bytes_observed_more_than_once, response_and_completion_evidence_present, idle_detection_uses_current_session_notifications, rollback_and_physical_stop_plan_ready] runtime_limits: one_active_brew: true one_write_at_a_time: true human_present: true unknown_frame_policy: abort disconnect_policy: abort_without_retry timeout_policy: abort_without_retry acceptance: source_integrity: "PASS when SHA-256 and byte count match manifest" privacy: "PASS when unrelated sessions and identifiers are removed" attribution: "PASS when every write belongs to one documented session" completeness: "PASS when start, completion and idle evidence all exist" repeatability: "PASS when repeated captures agree on required sequence" negative_cases: [wrong_recipe_is_rejected, missing_fragment_is_rejected, missing_response_is_not_success, disconnect_is_not_success, timeout_is_not_success, stale_notification_is_not_success, printer_failure_never_repeats_brew] report: status_values: [PASS, FAIL, INCOMPLETE] required_sections: [source_hashes, capture_windows, packet_evidence_index, observed_write_sequence, notification_state_machine, unknown_fields, negative_test_results, remaining_risks] final_notice: watermark_comment: >- STARHUB-NC-WATERMARK ¦ Copyright (c) 2026 StarHub / 翘翘 ¦ Non-commercial, written-authorized use only. prohibition: >- 严禁商业使用、转售、再分发、代运营或任何未经书面允许的使用。 ...